Stirring System

Equipment:

- 3V DC motor (maximum current 1A)

- CP2S700HCP photo-interrupter

- NPN BJT Transistor (ZTX450)

- Wires

- Arduino UNO

- 1N4001 diode

- 3V battery

How it works:

DC Motor:

- The base of the transistor is connected to pin 11 (PWM)and can control the speed of the motor.

- The emitter of the transistor is connected to ground.

- The collector is connected to the power supply.

- The diode is connected parallel to the dc motor in order to stop back emf from damaging the transistor and the Arduino due to a sudden opposite direction voltage increase.

- We ground the components for safety reasons.

Photo-interrupter:

- The photo-interrupter is connected to the Arduino at pin 3 (PWM) so it can send sensor values to the Arduino.

- It is connected to the 3.3V power supply of the Arduino and it is grounded to the Arduino GND.

Code:

- Motor is set as an OUTPUT (actuator) and the photo-interrupter as the INPUT (sensor).

- We set an initial output of motor (12% duty cycle = about 300 RPM).

- PID has a setpoint of 550 RPM, the controller begins to change the output to the motor after 5 seconds. By tuning the PID constant values we can change the rate at which the controller changes the output to the motor and control the oscillations between the setpoint.

- The photo-interrupter generates pulses when the propellers of the motor pass over it. We are finding out the time interval between each pulse. When the propeller goes over the sensor it generates zeros when it doesn’t it generates ones. The change between low to high is called a rising edge. In one full rotation we have 2 rising edges. RPM is the frequency*60. Therefore, in the code we have 60/(2*t(rising)).